## CPU Reset Procedures

This is a list of CPU resets for most ICOM radios. CPU reset returns the radio to out-of-box condition: all memory channels are erased, all items in the set mode returned to default values, and all VFOs cleared. CPU reset procedure usually involves turning the radio off, pushing and holding specific buttons, and then turning the radio back on while still holding down the button combination. Some newer radios have a "partial reset" available, as well as a full CPU reset. Partial reset will clear the VFOs and return the set mode items to default, but it will not erase the memory channels. Contents: Amateur radios CPU reset procedures Receivers CPU reset procedures

Amateur:

H A N D H E L D S: - IC-Delta1 - Hold [FUNCTION] & [A] & [CLR] buttons and turn the radio on. - IC-2GAT - Turn the radio on, hold [LIGHT] & [FUNCTION], turn the radio off, then turn back on. Release the buttons after the display returns to normal. - IC-2GXAT - Hold [FUNC] & [VMC] & [DUP] & [HI/LO] buttons and turn the radio on. - IC-02AT - Serial numbers below 34000: push the button on the main circuit board next to the lithium battery. Serial numbers above 34000: hold [FUNC] button and turn the radio on. - IC-2iA - Hold [LIGHT] & [MONITOR] buttons and turn the radio on. - IC-2SA - Hold [LIGHT] & [MONITOR] buttons and turn the radio on. - IC-2SAT - Hold [FUNCTION] & [A] buttons and turn the radio on. - IC-2SRA - Hold [FUNCTION] & [A] & [CLR] buttons and turn the radio on. - IC-P2AT - Hold [FUNCTION] & [A] buttons and turn the radio on. - IC-µ2AT - Hold [LIGHT] button and turn the radio on. - IC-03AT - Serial numbers below 34000: push the button on the main circuit board next to the lithium battery. Serial numbers above 34000: hold [FUNC] button and turn the radio on. - IC-3SA - Hold [LIGHT] & [MONITOR] buttons and turn the radio on. - IC-3SAT - Hold [FUNCTION] & [A] buttons and turn the radio on. - IC-4iA - Hold [LIGHT] & [MONITOR] buttons and turn the radio on. - IC-4GAT - Turn the radio on, hold [LIGHT] & [FUNCTION], turn the radio off, then turn back on. Release the buttons after the display returns to normal. - IC-4GXAT - Hold [FUNC] & [VMC] & [DUP] & [HI/LO] buttons and turn the radio on. - IC-04AT - Serial numbers below 34000: push the button on the main circuit board next to the lithium battery. Serial numbers above 34000: hold [FUNC] button and turn the radio on. - IC-4SA - Hold [LIGHT] & [MONITOR] buttons and turn the radio on. - IC-4SAT - Hold [FUNCTION] & [A] buttons and turn the radio on. - IC-4SRA - Hold [FUNCTION] & [A] & [CLR] buttons and turn the radio on. - IC-P4AT - Hold [FUNCTION] & [A] buttons and turn the radio on. - IC-µ4AT - Hold [LIGHT] button and turn the radio on. - IC-12AT - Hold [FUNC] button and turn the radio on. - IC-12GAT - Turn the radio on, hold [LIGHT] & [FUNCTION] buttons, turn the radio off, then turn back on. Release the buttons after the display returns to normal. - IC-24AT - Hold [FUNCTION] & [A] buttons and turn the radio on. - IC-32AT - Hold [FUNCTION] & [A] & [LIGHT] buttons and turn the radio on. - IC-Q7A - Hold [FUNC] & [BAND] & [V/M] buttons and turn the radio on. - IC-T2A - Hold [A] & [C] buttons and turn the radio on. - IC-T2H - Hold [aV] & [cT] buttons and turn the radio on. - IC-T21A - Full reset: hold [A] & [*] & [FUNC] buttons and turn the radio on. Partial reset: hold [A] button and turn the radio on. - IC-T22A - Full reset: hold [A] & [*] & [FUNC] buttons and turn the radio on. Partial reset: hold [A] button and turn the radio on. - IC-T41A - Full reset: hold [A] & [*] & [FUNC] buttons and turn the radio on. Partial reset: hold [A] button and turn the radio on. - IC-T42A - Full reset: hold [A] & [*] & [FUNC] buttons and turn the radio on. Partial reset: hold [A] button and turn the radio on. - IC-T7A - Hold [MR] & [VFO] & [BAND] buttons and turn the radio on. - IC-T8A - Hold [MR] & [VFO] & [BAND] buttons and turn the radio on. - IC-T81A - Hold [SQL] & [VFO] & [MR] buttons and turn the radio on. - IC-V21AT - Hold [F] & [A] & [*] buttons and turn the radio on. - IC-W2A - Hold [FUNCTION] & [A] & [CLR] buttons and turn the radio on. - IC-W21A - Hold [HI/LO] button and turn the radio on. - IC-W21AT - Hold [F] & [A] & [*] buttons and turn the radio on. - IC-W31A - Full reset: hold [A] & [*] & [MONI] buttons and turn the radio on. Partial reset: hold [A] button and turn the radio on. - IC-W32A - Full reset: hold [SQL] & [VFO] & [MR] buttons and turn the radio on. Partial reset: hold [VFO] button and turn the radio on. - IC-Z1A - Full reset: hold [A] & [*] & [MONI] buttons and turn the radio on. Partial reset: hold [A] button and turn the radio on. M O B I L E S: - IC-27A/H - Push the reset button under the top access cover. - IC-28A/H - Push the reset button in the aft left corner under the bottom cover. - IC-37A - Push the reset button under the top access cover. - IC-38A - Push the reset button in the aft left corner under the bottom cover. - IC-47A - Push the reset button under the top access cover. - IC-48A - Push the reset button in the aft left corner under the bottom cover. - IC-Delta100 - Full reset: hold [SET] & [SPCH] buttons and turn the radio on. Partial reset: hold [SPCH] button and turn the radio on. - IC-207H - Full reset: hold [SET] & [S.MW] buttons and turn the radio on. Partial reset: hold [V/MHz] button and turn the radio on. - IC-228A/H - Hold [SQUELCH/MONITOR] & [LOCK] buttons and turn the radio on. - IC-229A/H - Hold [SET] & [MW] buttons and turn the radio on. - IC-281H - Full reset: Hold [SET] & [SPCH] buttons and turn the radio on. Partial reset: hold [SPCH] button and turn the radio on. - IC-448A - Hold [SQUELCH/MONITOR] & [LOCK] buttons and turn the radio on. - IC-449A - Hold [SET] & [MW] buttons and turn the radio on. - IC-481H - Full reset: Hold [SET] & [SPCH] buttons and turn the radio on. Partial reset: hold [SPCH] button and turn the radio on. - IC-706 - Hold [UP] & [DOWN] buttons and turn the radio on. - IC-706MKII - Hold [UP] & [DOWN] buttons and turn the radio on. - IC-706MKIIG - Hold [UP] & [DOWN] buttons and turn the radio on. - IC-900 - Hold [MR] button, turn the power off and then on again. - IC-901A - Hold [CHECK] & [MW] buttons and turn the radio on. - IC-1200A - Hold down the tuning control and turn the power on. - IC-1201A - Hold [SQUELCH/MONITOR] & [LOCK] buttons and turn the radio on. - IC-2000H - Hold [SET] & [PGR/CS/MW] buttons and turn the radio on. - IC-2100H - Full reset: hold [SET] & [S.MW] buttons and turn the radio on. Partial reset: hold [V/MHz] button and turn the radio on. - IC-2340H - Full reset: hold [SET] & [SPCH] buttons and turn the radio on. Partial reset: hold [SPCH] button and turn the radio on. - IC-2350H - Full reset: hold [SET] & [S.MW] buttons and turn the radio on. Partial reset: hold [DUP] & [LOW] and turn the radio on. - IC-2400A - Hold [SUB VOL] & [MW] buttons and turn the power on. - IC-2410A/H - Hold [SET] & [MW] buttons and turn the radio on. - IC-2500A - Hold [SUB VOL] & [MW] buttons and turn the power on. - IC-2700H - Full reset: hold [SET] & [SPCH] buttons and turn the radio on. Partial reset: hold [SPCH] button and turn the radio on. - IC-2710H - Hold both [S.MW] buttons and turn the radio on. - IC-2800H - Full reset: hold 3rd buttons from the top on both sides of the LCD display and turn the radio on. Partial reset: hold 2nd buttons from the top on both sides of the LCD display and turn the radio on. - IC-3200A - Hold [F] button and turn the radio on. - IC-3210A - Hold [SQUELCH/MONITOR] & [LOCK] buttons and turn the radio on. - IC-3220A/H - Hold [SET] & [MW] buttons and turn the radio on. V H F / U H F B A S E: - IC-271A/H - Reprogram the RAM card. - IC-275A/H - Hold [M-CL] button and turn the radio on. - IC-375A - Hold [M-CL] button and turn the radio on. - IC-471A/H - Reprogram the RAM card. - IC-475A/H - Hold [M-CL] button and turn the radio on. - IC-575H - Hold [M-CL] button and turn the radio on. - IC-820H - Hold [MW] button and turn the radio on. - IC-821H - Hold [MW] button and turn the radio on. - IC-970A/H - Hold [MW] button and turn the radio on. - IC-1271 - Reprogram the RAM card. - IC-1275A - Hold [M-CL] button and turn the radio on. H F B A S E: - IC-77 - Clone with a computer. - IC-706 - Hold [UP] & [DOWN] buttons and turn the radio on. - IC-706MKII - Hold [UP] & [DOWN] buttons and turn the radio on. - IC-706MKIIG - Hold [UP] & [DOWN] buttons and turn the radio on. - IC-707 - Hold [MW] & [LOCK] buttons and turn the radio on. - IC-718 - Hold [UP] & [DOWN] buttons and turn the radio on. - IC-725 - Hold [FUNCTION] & [MW] buttons and turn the radio on. - IC-726 - Hold [FUNCTION] & [MW] buttons and turn the radio on. - IC-728 - Hold [FUNCTION] & [MW] buttons and turn the radio on. - IC-729 - Hold [FUNCTION] & [MW] buttons and turn the radio on. - IC-735 - No CPU reset available. - IC-736 - Hold [CLR] & [ENT] buttons and turn the radio on. - IC-737 - Hold [CLR] & [ENT] buttons and turn the radio on. - IC-737A - Hold [CLR] & [ENT] buttons and turn the radio on. - IC-738 - Hold [CLR] & [ENT] buttons and turn the radio on. - IC-746 - Hold [F-INP] & [M-CL] buttons and turn the radio on. - IC-751 - No CPU reset available. - IC-751A - No CPU reset available. - IC-756 - Hold [F-INP] & [M-CL] buttons and turn the radio on. - IC-756PRO - Hold [F-INP] & [M-CL] buttons and turn the radio on. - IC-761 - Hold [M-CLEAR] button and turn the radio on. - IC-765 - Hold [M-WRITE] button and turn the radio on. - IC-775 - Hold [CLEAR] button and turn the radio on. - IC-775DSP - Hold [CLEAR] button and turn the radio on. - IC-781 - Hold [M-CLEAR] button and turn the radio on. R E P E A T E R S: - IC-RP1220 - Turn the power off for a few minutes. Push S1 on the logic circuit board. - IC-RP1520 - Turn the power off for a few minutes. Push S1 on the logic circuit board. - IC-RP4020 - Turn the power off for a few minutes. Push S1 on the logic circuit board.

Receivers:

H A N D H E L D S: - IC-R1 - Hold [F] and [CL] buttons and turn the radio on. - IC-R2 - Hold [FUNC] & [BAND] & [V/M] buttons and turn the radio on. - IC-R10 - Full reset: hold [CLR] button and turn the receiver on. Rotate the main dial to select YES, then push [ENT] button. Partial reset: hold [MODE] button and turn the radio on. B A S E / M O B I L E: - IC-R71A - Reprogram the RAM card. - IC-R72 - Hold [MW] button and turn the radio on. - IC-R75 - Hold [UP] & [DN] buttons and turn the receiver on. - IC-R100 - Hold [FUNC] & [ENT] buttons and turn the radio on. - IC-R7100 - Hold [MW] button and turn the power on. - IC-R8500 - Hold [MW] button and turn the receiver on. - IC-R9000 - Hold [M-WRITE] button and turn the radio on.