GM3SEK in  BM 3000 hakkında ilave bilgileri

DC operating conditions for the GU-84B are:

* Va = 2.8-3.2kV on full load

* Ia_max = 1.2-1.5A

* Zero-signal anode current 400-450mA

* Vg2 = 375V

* Ig2 = 30-40mA normally, 50mA max (set current trip to 60mA). Negative 
screen current of 10-15mA may be seen in some conditions.

*Vg1 =  -60V, approximately, depending on the individual tube. The zener 
diode D114 should be 56V (1N5370) or maybe 60V (1N5371).

* Ig1_max = 1mA

Below the picture of the Tetrode Boards on RL3BM's web page, you will 
see some details of the mains transformer for all the "low" voltages. 
RL3BM is using a specially-manufactured toroidal transformer, but you 
can obtain the same voltages by using a number of different 
transformers.