Spools P-circuit: 
L1 - copper tube f6mm at bars f50mm, winding length of 120 mm, 11.5 turns (2.6 mkGn.). Disqualification of bands (from "hot" end):

28 - 3.5 spiral 
24/21 - 4.5 spiral 
18 - 5.5 spiral 
14 - 9.5 spiral

10 - 11.5 spiral (all coil) 
D2 - copper wire f2, 8mm at bars f50mm, winding length of 100 mm, 22 spiral (11.7 mkGn). Disqualification:

7 - 4 spiral 
3.5 - turns 12 
Aeq - constructive receptacle sockets 
C1, C3 - KVI-3 3300 h10Kv 
C2 - K15-5 4700 x6, 3Kv 
C4 - 12-60 PF 
P5 - 40-340 Pf 
C6 - 100. S7-150 K15U-1 6 kVA

C8, S9, S10 - 470 K15U-1 4 Kv 
C11, C12 - 680 K15U-2 2.5 Kv 
C13 - KVI-3 3300 h10Kv 
C14 - 50-960 pF 
Dr2 - turns 10 on 3 - x folded together rings K20 M2000NN 
T1 - secondary winding - 35-40 turns on a ring M20VCH K16 
Dr3 - 1 mGn

Separately - about Dr1. Anode throttles to be wound on ceramic tube f25mm length of 200 mm. PEV wire-0, 73 
The length of winding - 155 mm. Inside the tube is inserted ferrite rod f10mm M600NN length of 120 mm. Such construction anode throttles KK5DR detail in his article "Plate choke design" to http://www.kk5dr.com/chokes.htm