Command set for the KPC-9612

This is the command set for the KPC 9612 as described by the help menu.

COMMAND PARAMTERS DESCRIPTION

ABAUD   RS232 terminal baud rate

ANALOG   Displays analog inputs

AUTOLF   LF sent to terminal after CR

AX25L2V2   If on, AX25 version 2 protocol used

AXDELAY   Time added to TXDELAY for repeaters (10 msec)

AXHANG   Hang time of repeater to inhibit AXDELAY (10 msec)

BEACON  [{EVERY | AFTER}]  time (1 min)

BKONDEL   If on, DELETE will backspace; else \

BLT  n EVERY|AFTER hh:mm:ss [START hh:mm:ss]

BP96   If on, 9600 BT is .3; else BT is .5

BREAK   If on, a modem break will cause return to command mode

BTEXT   Text sent to BEACON in UI packet

BUDLIST  [ON|OFF] [NONE | {+|-}call | call,call,...]  where call={callsign | {<|>}callsign | callsign{>|<>}callsign} If on, only these stations are monitored

CONNECT  callsign [via calls]  can be used to reconnect with different path

CANLINE   Char used to cancel a partial line

CANPAC   Toggle char used to send output from TNC to bit bucket

CALIBRAT   immediate command to enter calibration mode

CD  {INTERNAL | EXTERNAL | SOFTWARE}  carrier detect

CHECK   No activity time before protocol checks on link (10 sec)

CMDTIME   Timing spec to get out of transparent mode (sec)

CMSG  {ON | OFF | DISC | PBBS}

COMMAND   Char used to return to command mode

CONLIST  [ON|OFF] [NONE | {+|-}callsign | callsign,callsign,..]  If on, only responds to stations in list

CONMODE  {CONVERS | TRANS}

CONOK   Enables connections to MYCALL

CONPERM   Forces connection on present stream to be permanent

CONVERS   Enter conversational mode

CPACTIME   Enables PACTIME use in conversational mode

CR   SENDPAC char appended to packets formed by SENDPAC

CRSUP   If on, removes every other consecutive CR received

CSTAMP   Time stamp connects and disconnects

CTEXT   Text sent first to someone else connecting to MYCALL

CTRL  {A|B} {OFF | ON | LONG | n}  controls digital output lines

CWID  [{EVERY | AFTER}]  time (1 min)

CWIDTEXT   Text sent during CW IDs

DISCONNE   Disconnect current stream

DAYTIME  yymmddhhmm[ss]

DAYTWEAK   Fine tune software 24hr clk

DAYSTR   time format

DAMA   DAMA Slave protocol used when on and connected to DAMA Master

DAMACHCK   No activity time before protocol checks on DAMA link (10 sec)

DBLDISC   Forces a true disconnect sequence after DISC while awaiting connect

DELETE   Char used to delete last char in line

DIGIPEAT   Allows digipeating through MYCALL

DISPLAY  {ASYNC | CHAR | GPS | ID | LINK | MONITOR | PBBS | PAGE | TIMING}

DWAIT   Time to allow digipeaters first access to channel (10 msec)

ECHO   Characters from terminal are echoed back

EQUALIZE   9600 BAUD EQUALIZATION

ESCAPE   If on, received ESCAPE characters printed as $

FLOW   TNC will not send data to terminal until input line complete

FILTER   If on, removes most control codes from received packets

FRACK   Time to wait for ack to packets (FRACK*(2*d+1) sec)

FULLDUP  {OFF | LOOPBACK | ON}  Allows transmitting while receiving

GPSHEAD  n  string

GPSINIT   Initial text sent to terminal in GPS mode

HELP  command[*]  prints help message

HBAUD  {300 | 400 | 600 | 1200 | 4800 | 9600 | 19200}  radio baud rate

HEADERLN   If on, prints a CR between monitored header and data

HID   Auto ID every 9.5 min if TNC is digipeating or PBBS or KA-Node used

HTEXT   Hierarchical address information

ID   Forces an ID packet

INTFACE  {TERMINAL | NEWUSER | BBS | GPS | HOST | KISS | XKISS | MODEM}

K   Same as CONVERS

KNTIMER   KA-Node idle timer

KNXCON   Allows the Xconnect command in KA-Node

LCOK   If on, allows lower case characters to be sent to terminal

LFADD   LF char added to each CR char sent in packet

LFSUP   If on, received LFs are not printed

LLIST  [ON|OFF] [NONE | {+|-}callsign | callsign,callsign,..]  If on, stations listed are completely ignored

LT   n text WILL SET LTn TEXT

LTP   n path SETS PATH FOR UI TRANSMISSION OF LTn

LTRACK  n [LT1] [LT2] [LT3] [LT4] [SYSOP] [TIME]

MONITOR   Master switch for monitoring packets

MALL   If on, allows monitoring of connected data between other stations

MAXFRAME   Maximum number of unacknowledged I frames outstanding

MAXUSERS   Maximum number of connections allowed

MBEACON   If on, monitors packets addressed to BEACON and ID

MCON   If on, allows monitoring to continue while connected

MCOM   If on, monitors SABM (), DISC, DM, and UA frames

MHEARD  [LONG|SHORT|CLEAR]  Display list of stations heard

MHEADER   If on, headers are printed for monitored packets

MRESP   If on, monitors RR, RNR, REJ frames

MRPT   If on, digipeater paths are displayed in header

MSTAMP   If on, monitored packets are time stamped

MXMIT   If on, transmitted packets are monitored

MYCALL   This station's callsign {call[-n]}

MYALIAS   Allows digipeating with alias callsign

MYDROP   KISS port address

MYGATE   Allows cross port digipeating with gate callsign

MYNODE   Callsign used for KA-Node

MYPBBS   Callsign used for access to personal mail box

MYPAGE   Callsign allows access to pager

MYREMOTE   Callsign allows access to remote control

NDHEARD  [LONG|SHORT|CLEAR]  Display list of NODES heard

NDWILD   Allows connection to any ssid of MYNODE

NEWMODE   If on, returns to command mode when DISC received on current stream

NOMODE   If on, does not automatically change modes between CONV, TRANS, or CMD

NTEXT   Text sent to someone connecting to KA-Node

NUMNODES   Number of KA-Node channels

ONERADIO   Both ports used on one radio

PACLEN   Maximum size of data in I frame

PACTIME  [{EVERY | AFTER}] time (.1 sec)  when to form packets in trans mode

PAGE  [-A|-N] [-512|-1200|-2400] [ message

PAGECWID  n|ON|OFF  if on, CWID sent after every page, else after n min

PAGEDIR  n | {LIST [call|alias|capcode]} | {+|- callsign [alias] [-A|-N] [-512|-1200|-2400] [-P] capcode}

PAGELOG  n | LIST | CLEAR

PAGEMON  ON|OFF|nzza  Enables monitoring of pages and defines type

PAGEPRIV   Restricts PAGE SERVER to password access

PAGEPSWD   Text string used for paging authorization

PAGETEXT   Initial message sent to someone connecting to MYPAGE

PAGEXINV   Inverts polarity of transmitted pages

PASS   Char used to pass special characters as data

PASSALL   Allows packets with incorrect CRCs to be processed

PBBS   Amount of memory (1K increments) set aside for PBBS messages

PBFORWRD  [bbscall [vias]] [PORT{1|2}] [{EVERY|AFTER} n]  starts auto forwarding

PBHEADER   Saves routing information in messages

PBHOLD   Incoming messages to PBBS are marked HOLD

PBKILLFW   Kill Personal and Traffic msgs after forwarding

PBLIST  [ON|OFF] [NONE | {+|-}callsign | callsign,callsign,..]  If on, PBBS considers list as family

PBLO  [{OLD | NEW}] [{FIXED | VARIABLE}]  PBBS listing order

PBPERSON   Restricts PBBS messages addressed to MYCALL and MYPBBS

PBREVERS   Accepts messages after AutoForwarding

PERSIST   Probability of transmitting during slot time (0-255)

PHEARD  [CLEAR]  Display list of PBBS users

PID   If on, allows monitoring of all packets regardless of PID

PMODE  CMD | CONVERS | TRANS  TNC powers up in this mode

PORT  {1|2}  Port assigned on power up

PTEXT   Initial text sent when someone connects to PBBS

RESET   Soft reset

REDISPLA   Char used to redisplay partial line

RELINK   Version 2 will try to reconnect after retrying out

RESTORE   DEFAULTS

RETRY   Number of retries before aborting

RING   3 bells sent to terminal upon incoming connect

RNRTIME   TNC will disconnect after receiving RNRs for specified time (x10 sec)

RPRINT   Sends text to RS232 port

RTEXT   Text string used for remote authorization

STATUS  [LONG]  Show status of streams

SCREENL   CR printed if none received after n characters

SENDPAC   Char used to form packets in convers mode

SLOTTIME   Time between successive tries of persistance algorithm (10 msec)

START   Char to restart output from TNC to terminal

STOP   Char to stop output from TNC to terminal

STREAMSW   Char used to change streams

STREAMCA   Show callsigns with stream switch

STREAMEV   Show stream switch with every packet

SUPLIST  [ON|OFF] [NONE | {+|-}call | call,call,...]  where call={callsign | {<|>}callsign | callsign{>|<>}callsign} If on, stations in list are not monitored

SWP   Allows fine tuning of SOFTWARE CD

TRANS   Enter transparent mode

TRACE   Display received packets in hex

TRFLOW   If on, allows software flow control in TRANS receive mode

TRIES   Number of retries on current packet

TXDELAY   Time delay between PTT and radio data out (10 msec)

TXFLOW   If on, allows software flow control in TRANS transmit mode

UNPROTO  {call [via calls]}  path for UI data frames

USERS   Specifies streams to be used by incoming connects

VERSION   Displays firmware version number

ID   Forces an ID packet

INTFACE  {TERMINAL | NEWUSER | BBS | GPS | HOST | KISS | XKISS | MODEM}

XFLOW   Master switch for software flow control

XKCHKSUM   If on, Extended KISS uses a checksum

XKPOLLED   If on, TNC must be polled in Extended KISS

XMITLVL   Transmit levels (0-502)/(0-255)

XMITOK   If on, allows TNC to key transmitter

XOFF   Char to stop input from terminal to TNC

XON   Char to restart input from terminal to TNC